# Travel Trailer Winter Storage Checklist

# 1. Drain and Winterize the Plumbing System

- ☐ Empty all water tanks: Drain the fresh water tank and flush both gray and black tanks completely. Use a tank cleaning wand or built-in flush system if available.

- ☐ Drain the water heater: Turn off the power and allow it to cool before draining.

- ☐ Bypass the water heater: Install a bypass kit if not already in place to avoid wasting antifreeze in the tank.

- ☐ Blow out water lines: Use an air compressor with a blow-out plug (30-40 PSI max) to remove residual water from faucets and lines.

- ☐ Pump in RV antifreeze: Using a water pump converter kit, push non-toxic pink RV antifreeze through all water lines, including kitchen and bathroom sinks, showerheads, outdoor showers, and toilets.

- ☐ Don’t forget the traps: Pour a cup of antifreeze into each drain and the toilet bowl to protect the P-traps and seals.

- 🧴 Product Tip: Use Camco RV Antifreeze Concentrate or RecPro Ready-to-Use RV Antifreeze.

## 2. Interior Preparation

- ☐ Remove all food: Even unopened food can attract rodents.

- ☐ Defrost and clean the fridge: Leave fridge and freezer doors cracked open to prevent mold.

- ☐ Clean appliances and floors: A deep clean now prevents surprises later.

- ☐ Remove soft goods: Take home blankets, towels, pillows, and anything that can retain moisture or odor.

- ☐ Open cabinets and drawers: Allows air circulation and reduces musty smells.

- ☐ Moisture control: Use DampRid, Eva-Dry dehumidifiers, or silica gel packs in corners and closets.

- 🧽 Pro Tip: Toss in a few dryer sheets to keep things smelling fresh and help deter mice.

## 3. Exterior Maintenance

- ☐ Wash and wax: Remove road grime, bugs, and sap, then apply a protective wax.

- ☐ Inspect all seals and seams: Check windows, roof vents, and doors for cracks. Reseal with Dicor Lap Sealant or Geocel ProFlex.

- ☐ Lubricate moving parts: Hinges, locks, stabilizer jacks, and the tongue jack all appreciate a little lube before the freeze.

- ☐ Cover vents and pipes: Use mesh covers or foam to keep critters out.

- ☐ Check and inflate tires: Inflate to the recommended PSI and consider using tire covers or placing boards under tires to keep them off cold pavement.

- 🛞 Product Tip: Moonet RV Tire Covers are a great and affordable option.

## 4. Battery and Electrical Systems

- ☐ Charge fully before storage: A fully charged battery is less likely to freeze.

- ☐ Disconnect and remove: Store in a dry location. Keep them off concrete floors and away from freezing temps.

- ☐ Use a maintainer: Battery tenders or solar trickle chargers help maintain battery health.

- ☐ Shut off main breaker: Prevents phantom power draw and unexpected drain.

- ☐ 🔌 Product Tip: NOCO Genius1 Smart Battery Charger or a 5W solar battery maintainer for long-term trickle charging.

## 5. Final Touches Before You Walk Away

- ☐ Cover your RV: Use a breathable RV cover to prevent moisture buildup while protecting from sun, snow, and debris.

- ☐ Block entry points: Use steel wool or pest deterrents in common mouse entrances (electrical cord holes, vents).

- ☐ Secure stabilizers: Retract or snug them down with blocks, but don’t lift the trailer.

- ☐ Lock doors and windows: Obvious but essential.

- ☐ Leave a checklist inside: Add a 'de-winterizing' checklist so you're ready to roll when spring arrives.

- 🧯 Bonus Tip: Check fire extinguisher expiration and smoke/CO detectors while you’re at it.